國  立  清  華  大  學

碩  士  論  文

聲學傳遞函數盲估計以應用於去混響、聲源分離以及增強

Blind estimation of acoustic transfer functions with application to signal dereverberation, source separation, and speech enhancement

系級：動力機械工程學系碩士班

組別：電機控制組

學號姓名：111033537 袁安志 Anchi Yuan

指導教授：白明憲 博士 (Dr. Mingsian R. Bai)

中 華 民 國 一 一 三 年 七 月

- 摘要

雖然在陣列信號處理中，聲學傳遞函數（Acoustic Transfer Functions）通常比相對傳遞函數（Relative Transfer Functions）具有更好的性能，但由於源輸入信號通常不可用，獲得可靠的聲學傳遞函數估計具有挑戰性。為了解決這一問題，我們提出了一種基於卷積傳遞函數（Convolutive Transfer Functions）的創新盲聲學傳遞函數估計方法。我們首先使用到達時間差（Time Difference of Arrival）和廣義互相關相位變換（Generalized Cross Correlation-Phase Transform）估計來定位分佈式陣列中的聲源。接著，我們應用加權預測誤差（Weighted Prediction Error）算法對混合緊湊-分佈式陣列接收到的信號進行去混響，並使用延遲和求和波束形成器作為源信號的初步估計。卷積傳遞函數係數可以使用維納濾波器或卡爾曼濾波器計算，並使用粒子群優化（Particle Swarm Optimization）優化其參數。數值模擬和使用十三麥克風混合陣列進行的實驗證明了所提出技術的有效性。最先進的自適應多通道時域最小均方（Adaptive Multi-channel Time Domain Least Mean Square）方法被用作基線。為了進一步驗證，我們將所提出的方法應用於信號去混響、聲源分離和語音增強等應用。

關鍵詞 ― 卷積傳遞函數，加權預測誤差算法，延遲和加總波束成形器，維納濾波器，卡爾曼濾波器，粒子群優化

- ABSTRACT

While Acoustic Transfer Functions (ATFs) generally lead to better performance than Relative Transfer Functions (RTFs) in array signal processing, obtaining reliable ATF estimates is challenging because the source input is usually unavailable. To address this problem, we propose a novel blind ATF estimation approach formulated using Convolutive Transfer Functions (CTFs). We start by locating the source using Time Difference of Arrival (TDOA) estimated by Generalized Cross Correlation-Phase Transform (GCC-PHAT), by using a distributed array. Next, we apply the Weighted Prediction Error (WPE) algorithm to dereverberate the signals received by a hybrid compact-distributed array, using the Delay and Sum beamformer as an initial estimate of the source signal. The CTF coefficients can be computed using either the Wiener filter or the Kalman filter with the parameters optimized using Particle Swarm Optimization (PSO). Simulations and experiments using a thirteen-microphone hybrid array demonstrate the efficacy of the proposed technique. The state-of-the-art Adaptive Multichannel Time Domain Least Mean Square (MCLMS) method was used as the baseline. For further validation, we applied the proposed technique to applications, including signal dereverberation, source separation, and speech enhancement.

Index Terms — convolutive transfer functions, weighted prediction error, delay and sum beamformer, Wiener filter, Kalman filter, particle swarm optimization

- CONTENTS

摘要	ii

ABSTRACT	iii

CONTENTS	v

LIST OF ALGORITHMS	vii

LIST OF FIGURES	viii

LIST OF TABLES	ix

Chapter 1.	INTRODUCTION	1

Chapter 2.	BASELINE APPROACH	5

Chapter 3.	CTF SIGNAL MODEL	11

3.1. Representation of LTI Systems in Crossband Filter	11

3.2. Band-to-band Filter as CTF Signal Model	14

Chapter 4.	PROPOSED METHOD	18

4.1. Source Localization	18

4.2. Pre-processing	18

4.2.1. WPE	18

4.2.2. DAS Beamformer	22

4.3. Wiener Filtering Approach	22

4.4. RLS Approach	24

4.5. Kalman Algorithm Adaptive Filtering Approach	25

4.6. ATF Reconstruction	27

4.7. Parameters Optimization	28

4.7.1. PSO	28

4.7.2. ASPSO	30

4.8. Summary of Proposed Method	35

Chapter 5.	SIMULATIONS	37

5.1. Simulation Settings and Parameters	37

5.2. Simulation Results and Discussions	38

5.2.1. Without Parameters Optimization	38

5.2.2. With Parameters Optimization	44

Chapter 6.	EXPERIMENTS	46

6.1. Experiment Settings and Parameters	46

6.2. Experiment Results and Discussions	46

Chapter 7.	CONCLUSIONS AND FUTURE WORK	46

7.1.	Conclusions	46

7.2.	Future Work	47

REFERENCES	48

To do list	53

- LIST OF ALGORITHMS

Algorithm 1 WPE	21

Algorithm 2 CTF estimation using Wiener filtering	24

Algorithm 3 CTF estimation using RLS	25

Algorithm 4 CTF estimation using stationary Kalman adaptive filtering	26

- LIST OF FIGURES

Figure 1 Block diagram of PSO	30

Figure 2 Block diagram of ASPSO	35

Figure 3 Configuration of the room for simulations	38

Figure 4 Absolute error of the estimated ATF and magnitude of the estimated RIR when T60 = 0.6s	41

Figure 5 ME of the estimated ATFs based on the proposed methods in various reverberation times	42

Figure 6 The PESQ values of the processed and unprocessed signals at different reverberation time	43

Figure 7 The SDR values of the processed and unprocessed signals at different reverberation time	44

- LIST OF TABLES

Table 1 Flow chart of our proposed method	35

Table 2 ME of the estimated ATFs based on the proposed methods in various reverberation times	41

Table 3 ME of the estimated ATFs with and without optimization at T60 = 0.6s	45

INTRODUCTION

Blind estimation, namely blind system identification (BSI), pertains to identifying systems where solely the output signals are known, and a minimal amount of data is known regarding the input signals. This challenge is highly significant due to the practical applications of estimated Acoustic Transfer Functions (ATFs) in various acoustic scenarios such as acoustic echo cancellation [1] , dereverberation [2] , blind source separation [3] , and beamforming in reverberant environments [4] . Most BSI techniques have typically functioned using the time domain [5] or the Short Time Fourier Transform (STFT) domain [6] [7] , where they estimate convolution in the time domain by multiplying the source STFT with the room impulse response (RIR) STFT. This approximation, called the multiplicative transfer function (MTF) approximation [8] or the narrowband approximation, is valid in theory only if the RIR's length is shorter than that of the STFT window. However, in practical scenarios, this requirement is rarely met, even in moderately reverberant environments. The limitations of the STFT window in assuming local stationarity of audio signals result in this. Additionally, the use of a long STFT window can lead to increased estimation variance and computational complexity.

To tackle this problem, especially in situations involving extended RIRs, Crossband Filters (CBFs) were introduced in [9] for linear system identification. These CBFs provide an alternative to the MTF approach. In this alternative, the STFT coefficient output is represented as the sum of multiple convolutions between the STFT coefficients of the input source signal and the RIR in the time-frequency (TF) domain along the frame coordinate. For analytical tractability, an approximation of CBFs called the convolutive transfer function (CTF) [10] has been proposed. This model proposes that, for each frequency, the STFT coefficient output can be represented as a distinct convolution between the STFT coefficients of the input source signal and the CTF along the frame axis.

This thesis outlines a methodology for the estimation of blind ATF through CTF approximation. To start the process, source localization algorithm is needed to obtain accurate source location in the space for the source signal extraction of the next stage. We, in this thesis, use Generalized Cross Correlation-Phase Transform (GCC-PHAT) [11] to estimate Time Difference of Arrival (TDOA) [12] of each microphone in a distributed array. Once source location was acquired, we can move to dereverberation and source signal extraction using Weighted Prediction Error (WPE) [13] and Delay and Sum (DAS) beamforming [14] . Afterward, CTF coefficients are computed using Wiener filters [15] or adaptive filters, such as Recursive Least Squares (RLS) [16] and Kalman filter [17] . The parameters of previous mentioned filters are optimized using Particle Swarm Optimization (PSO) [18] and its enhanced version [19] . To acquire ATFs in the time domain, namely Room Impulse Responses (RIRs), from the CTF coefficients, the estimated CTF coefficients are convolved with the constant magnitude filter across frequency axis. Subsequently, the resulting convolved sequence is subjected to processing using the inverse STFT.

The convergence performance is assessed using the normalized root mean square projection misalignment (NRMSPM) between the ground truth RIR and the estimated RIR. The results are compared with the baseline approach, specifically the Adaptive Multichannel Time Domain Least Mean Square (MCLMS) [20] method. Simulations encompass a wide range of reverberation times from 0.01 seconds to 1.6 seconds, utilizing a hybrid compact-distributed array of 38 microphones. Applications include signal dereverberation using the Multiple Input/Output Inverse Theorem (MINT) [21] , source separation via Tikhonov Regularization (TIKR) [22] , and speech enhancement using the Minimum Power Distortionless Response (MPDR) [23] beamformer, all of which require precise ATF estimates. The effectiveness of these applications is evaluated using several metrics, including the Perceptual Evaluation of Speech Quality (PESQ) [24] and Signal-to-Distortion Ratio (SDR) [25] . Additionally, experiments are conducted in a realistic room with a reverberation time of 0.128 seconds, using a hybrid compact-distributed array of 13 microphones. The results from both simulations and experiments demonstrate that the proposed approach outperforms the MCLMS method in various reverberant environments.

BASELINE APPROACH

In this section, we introduce a state-of-the-art blind system identification algorithm known as Adaptive Multichannel Time Domain Least Mean Square (MCLMS), which we adopt as a baseline approach to compare with our proposed ATF estimation method. This approach constructs an error signal based on the cross-relations between different channels in a novel and systematic manner, as detailed below.

The i-th observation xi(n) is the result of a linear convolution between the source signal s(n) and the corresponding channel response hi. This relationship is expressed as follows:

| , | ( 1 ) |
| --- | --- |

where the * indicates linear convolution with respect to the time index n and M is the number of channels. In vector form, the relationship between the input and the observation for the i-th channel is written as:

| , | ( 2 ) |
| --- | --- |

where

| , | ( 3 ) |
| --- | --- |

and T denotes matrix transpose. The channel parameter matrix Hi is of dimension L × (2L - 1) and is constructed from the channel’s impulse response:

| , | ( 4 ) |
| --- | --- |

where L is set to the length of the longest channel impulse response by assumption.

When the input signal is unknown, the cross-relation between sensor outputs can be utilized to estimate the channel impulse responses. This is based on the premise that

| . | ( 5 ) |
| --- | --- |

However, during prediction, this cross-relation does not hold, leading to the definition of an error signal as follows:

| . | ( 6 ) |
| --- | --- |

We have (M - 1)M/2 distinct error signals eij(n) , excluding the case where eii(n) = 0 and counting the pair eij(n) = -eji(n) only once. Assuming the equal importance of these error signals, we define a cost function as:

| . | ( 7 ) |
| --- | --- |

The channel impulse responses are then defined by minimizing this error function. To avoid a trivial all-zero estimate, a unit-norm constraint is imposed on h = [h1T h2T…hMT]T at all times, such that the error signal becomes:

| . | ( 8 ) |
| --- | --- |

The corresponding cost function is given by:

| . | ( 9 ) |
| --- | --- |

The desired solution for h is determined by minimizing the mean value of the cost function J(n):

| . | ( 10 ) |
| --- | --- |

Direct minimization is a computationally intensive process that may be intractable for long channel impulse responses and a large number of channels. Consequently, an LMS algorithm is proposed as a solution to this minimization problem, offering an efficient solution:

| , | ( 11 ) |
| --- | --- |

where μ represents a small positive step size, while ∇ denotes the gradient operator. In order to ascertain the gradient in ( 11 ) , it is necessary to take the derivative of J(n) with respect to h:

| , | ( 12 ) |
| --- | --- |

where

| . | ( 13 ) |
| --- | --- |

We will now evaluate the partial derivative of χ(n) with respect to the coefficients of the k-th (k = 1,2, …, M) channel impulse response:

| , | ( 14 ) |
| --- | --- |

where the final step follows from the fact that ekk(n) = 0. This equation may be expressed in matrix form concisely as follows:

| , | ( 15 ) |
| --- | --- |

where we have defined, for the sake of convenience,

| . | ( 16 ) |
| --- | --- |

Subsequently, the two matrix products in ( 15 ) are evaluated individually as follows:

| , | ( 17 ) |
| --- | --- |

where

| . | ( 18 ) |
| --- | --- |

Substitution of ( 17 ) into ( 15 ) yields the following result:

| . | ( 19 ) |
| --- | --- |

Subsequently, we integrate ( 19 ) into ( 12 ) and derive the following result:

| , | ( 20 ) |
| --- | --- |

where

| . | ( 21 ) |
| --- | --- |

Finally, we substitute ( 20 ) into ( 11 ) and obtain the updated equation:

| . | ( 22 ) |
| --- | --- |

If the channel estimate is always normalized following each update, then the simplified algorithm can be implemented as follows:

| . | ( 23 ) |
| --- | --- |

Assuming that the independence assumption [26] holds, it can be demonstrated that the LMS algorithm converges in the mean if the step size satisfies

| , | ( 24 ) |
| --- | --- |

where the largest eigenvalue of the matrix E{Rn-J(n)IMLxML} is denoted by λmax.

CTF SIGNAL MODEL

3.1. Representation of LTI Systems in Crossband Filter

In this section, we provide a concise overview of how digital signals and LTI systems are represented in the STFT domain. For more extensive information, please refer to sources such as [27] and [28] . First, we establish links between the crossband filters in the STFT domain and the impulse response in the time domain using analysis and synthesis windows. Unless stated otherwise, our summation indexes range from -∞ to ∞.

The STFT representation of a signal x(n) is given by

| , | ( 25 ) |
| --- | --- |

where

| . | ( 26 ) |
| --- | --- |

An analysis window of length N is denoted by w̃(n). The frame index is denoted by p ∈ [1, P], and k ∈ [0, N–1] represents the frequency-band index. The discrete-time shift is denoted by Ls. Complex conjugation is represented by *. The reconstruction of x(n) which is inverse STFT is achieved by

| , | ( 27 ) |
| --- | --- |

where

| , | ( 28 ) |
| --- | --- |

and w(n) denotes a synthesis window of length N. This thesis assumes w̃(n) and w(n) are real functions. By substituting ( 25 ) into ( 27 ), we acquire the completeness condition

| . | ( 29 ) |
| --- | --- |

If the analysis and synthesis windows meet the requirements outlined in ( 29 ), the signal x(n) can be reconstructed flawlessly using its STFT coefficients xp,k. However, for Ls≦N and for a given synthesis window w(n), there might be an infinite number of solutions to ( 29 ); thus, the choice of the analysis window may not be unique according to [29] and [30] .

We will now delve into the STFT representation of LTI systems. Let h(n) denote the impulse response of an LTI system with a length of Q, where the input x(n) and output o(n) of this system are connected through the relation as follow:

| . | ( 30 ) |
| --- | --- |

From ( 25 ) and ( 30 ), the STFT of o(n) can be written as

| . | ( 31 ) |
| --- | --- |

Substituting ( 27 ) into ( 31 ), we obtain

| , | ( 32 ) |
| --- | --- |

where

|  | ( 33 ) |
| --- | --- |

may be interpreted as the STFT of h(n) using a composite analysis window ∑m wpˊ,kˊ(m-l) w̃p,k*(m). Substituting ( 26 ) and ( 28 ) into ( 33 ), we obtain

| , | ( 34 ) |
| --- | --- |

where * indicates linear convolution with respect to the time index n, and

| . | ( 35 ) |
| --- | --- |

From ( 34 ), we know that hp,k,pˊ,kˊ depends on (p - pˊ) rather than on p and pˊ separately. Substituting ( 34 ) into ( 32 ), we obtain

| . | ( 36 ) |
| --- | --- |

From ( 34 ), we also obtain

| . | ( 37 ) |
| --- | --- |

From ( 35 ), we get

| , | ( 38 ) |
| --- | --- |

where wn,k is the STFT representation of the synthesis window w(n) calculated with a decimation factor Ls=1. Equation ( 36 ) demonstrates that for a particular frequency-band index k, the temporal signal can be acquired by convolving the signal xp,kˊ in each frequency-band kˊ(kˊ= 0,1,…,N-1) with its corresponding filter hp,k,kˊ and subsequently adding up all the outputs. Here, the term for k = kˊ is referred to as a band-to-band filter, and k ≠ kˊ is referred to as a crossband filter, and crossband filters are employed to eliminate the aliasing effects resulting from subsampling. Note that ( 37 ) indicates that, in general, for fixed k and kˊ, the filter hp,k,kˊ has ⌈N/ Ls ⌉ – 1 non-causal coefficients. Hence, in echo cancellation applications, these coefficients must be taken into consideration. Extra delay of (⌈N/ Ls ⌉ – 1) Ls samples is typically introduced into the microphone signal to deal with this problem, as illustrated in [31] .

3.2. Band-to-band Filter as CTF Signal Model

In this paragraph, we will derive a CTF signal model for blind ATF estimation using band-to-band filters.

In a noise-free and reverberant environment, a speech signal transmits to microphones via the room effect. In the time domain, the received source image y(n) is specified by

| , | ( 39 ) |
| --- | --- |

where s(n) and a(n) represent the source signal and the RIR, respectively, with * indicating linear convolution with respect to time index n. The RIR in ( 39 ) is often estimated using MTF in the STFT domain, as demonstrated by

| , | ( 40 ) |
| --- | --- |

where yp,k and sp,k represent the STFTs of their respective signals, while ak denotes the Fourier transform of the RIR a(n). In addition, p ∈ [1, P] refers to the frame index, N indicates the STFT window size, and k ∈ [0, N–1] represents the frequency index as in crossband filter. However, it is important to note that the approximation in ( 40 ) is accurate only if the length of the RIR a(n) is shorter than the STFT window size N. In real-world scenarios, numerous filter taps must be taken into account, numbering in the thousands, resulting in a destroyed approximation. Therefore, a significant increase in computational complexity and a slow convergence rate will occur. To overcome this problem, the crossband filter model is used in this study. From ( 36 ) the STFT coefficient yp,k is expressed as the sum of several convolutions between the STFT-domain source signal and the filter across the frequency bins, as follows:

| . | ( 41 ) |
| --- | --- |

Assuming D is the STFT frame step as stated above, if D is less than N, then apˊ,k,kˊ becomes non-causal, with ⌈N/D⌉ – 1 non-causal coefficients. The number of causal filter coefficients is dependent on the reverberation time. For simpler notation, we assume that the filter index pˊ ranges from 0 to L –1, where L is the length of the filter. This requires shifting the non-causal coefficients to the causal component, which leads to a fixed delay shift of ⌈N/D⌉ – 1 of the frame index for the received microphone signal [31] . From ( 37 ) the STFT domain impulse response apˊ,k,kˊ relates to the time domain impulse response a(n) by

| , | ( 42 ) |
| --- | --- |

which indicates the convolution with respect to the time index n evaluated at frame steps using ( 38 ). Note that for the remainder of this article, we will continue to refer to the analysis and synthesis windows in the STFT procedure as w̃(n) and w(n), respectively. To streamline the analysis, we employ the so called CTF approximation, which focuses exclusively on the band-to-band filters with k = kˊ, as described in the following:

| . | ( 43 ) |
| --- | --- |

Based on this, we are considering a version with multiple channels of M microphones

| , | ( 44 ) |
| --- | --- |

where yi p,k and ai p,k represent the i-th microphone signal and the corresponding CTF, respectively. Therefore, the source signals can be expressed in matrix form as follow:

| . | ( 45 ) |
| --- | --- |

Since the proposed algorithm functions on a frequency basis, the frequency index will be omitted henceforth for the sake of brevity. From ( 45 ) we can rewrite the matrix form with respect to frame index as follow:

| , | ( 46 ) |
| --- | --- |

where the bold symbols denote vectors or matrices and the subscript d denotes the delayed signal. Up to this point, we have derived the CTF signal model, which corresponds to equation ( 46 ).

PROPOSED METHOD

In this section, we introduce a technique for estimating the ATF in a blind manner. It should be emphasized that we possess solely the positions of the microphones and the source, alongside the delayed microphone signal yd,p (by ⌈N/D⌉ – 1 frames [31] ) and pre-processed source signal acquired via the non-delayed microphone signal yp, which will undergo processing with the WPE and DAS beamformer. Notably, we provide three techniques for estimating the CTF coefficients: the Wiener filter, RLS and Kalman adaptive filter.

4.1. Source Localization

4.2. Pre-processing

In order for the subsequent CTF estimation algorithm to function effectively, it is necessary to have access to clean and echo-free source signal. However, in practical applications, obtaining clean source signal is often challenging. Therefore, in this thesis, we employ the Weighted Prediction Error (WPE) method for dereverberation [13] . Subsequently, we utilize the Delay and Sum (DAS) beamformer [14] to extract clean source signal from the WPE outputs of all channels.

4.2.1. WPE

If only one speech source is captured by M microphones, then we may rewrite ( 39 ) as follows:

| , | ( 47 ) |
| --- | --- |

where m and La represent the ordinal numbers of the m-th microphone and length of the RIR. The reverberant signal ym(n) in ( 47 ) consists of three parts: a direct signal, early reverberation, and late reverberation. It is common practice to take the first two components as the desired signal, which is denoted by dm(n). Meanwhile, the late reverberation is taken as the signal to be eliminated and is denoted by rm(n). The relationship between these signals can be expressed as such:

| , | ( 48 ) |
| --- | --- |

where

| , | ( 49 ) |
| --- | --- |

where C is the sample index that distinguishes the RIR into the early and late reverberation parts. This index is subsequently referred to as the prediction delay. From now on, we assume that there two microphones, namely M = 2, for the sake of simplicity. If the RIRs am(n) in different channels do not share common zeros, the relationship between speech signal and the microphone signals in ( 47 ) can be rewritten (stepwise derivation is shown in [32] ) as

| , | ( 50 ) |
| --- | --- |

where T denotes matrix transpose and

| , | ( 51 ) |
| --- | --- |

where cm and Lc indicate the vector of regression coefficients and the regression order, respectively, and LH equals Lc + La – 1.

Using the estimated vector of regression coefficients cm and following ( 50 ), it is possible to acquire the desired signal as follow:

| . | ( 52 ) |
| --- | --- |

Hence, the dereverberation can be achieved by obtaining a suitable estimated vector of regression coefficients cm from the microphone signals. Because c1 is completely determined independently of c2, in the following, we disregard the optimization of c2 without loss of generality. The resultant optimization algorithm can be summarized (stepwise derivation is shown in [13] ) as follow.

Algorithm 1 WPE

Input: y1(n), y(n) - Initialize σ(n)2 as ,; ( 53 ). where Lf is length of short time frame and μ > 0 is a certain lower bound to avoid zero division. - Repeat the following steps until convergence. - Update c1 as follows: ,; ( 54 ). where + denotes the pseudo inverse and ( 55 ). ,; ( 56 ). where τ is the largest sample index of the microphone signal. - Update d1(n) as d1n= y1n-(c1)Ty(n-D). - Update σ(n)2 as follow: .; ( 57 ).

It is worth noting that we will refer to the WPE output of the m-th channel dm(n) as yWPEm(n) from this point forward.

4.2.2. DAS Beamformer

Once the dereverberated microphone signal yWPEm(n) from WPE is obtained, clean source signal extraction through DAS beamformer can be executed. First, we convert yWPEm(n) to the STFT domain, resulting in yWPE,pm. Here, p still represents the frame index as previously explained. Secondly, we calculate the beamforming weight of the DAS beamformer as follow:

| , | ( 58 ) |
| --- | --- |

where к and Rm denote wave number and distance between source and m-th microphone, respectively. Finally, we perform the inner product between the weights and yWPE,pm to get clean source signal as follow:

| , | ( 59 ) |
| --- | --- |

where H denotes Hermitian transpose.

4.3. Wiener Filtering Approach

For the first technique, the Wiener-based derivations are employed to estimate the matrix of CTF coefficients A. This approach minimizes the mean square error as follow:

| , | ( 60 ) |
| --- | --- |

where E[·] denotes the expectation with respect to the frames. Therefore, ( 60 ) can be rewritten as

|  | ( 61 ) |
| --- | --- |

where tr{·} denotes the matrix trace, and the associated covariance matrices is

| . | ( 62 ) |
| --- | --- |

By taking the derivative of ( 61 ) with respect to , we obtain

| . | ( 63 ) |
| --- | --- |

The optimal Wiener solution can be obtained as

| . | ( 64 ) |
| --- | --- |

In practical implementation, instead of the expectation, the recursive averaging is adopted to obtain Rsy and Rss as given by

| , | ( 65 ) |
| --- | --- |

where α denotes the forgetting factor for the recursive averaging process. The Wiener filtering approach can be summarized as follows.

Algorithm 2 CTF estimation using Wiener filtering

Input: yd,p, sDAS,p - Initialize forgetting factor α and covariance matrices as - For each instant of frame, p = 1, 2, …, compute .; ( 66 ).

It should be noted that the estimated matrix of CTF coefficients A changes depending on the processing frame, and the accuracy improves with an increase in the number of processed frames.

4.4. RLS Approach

For the second technique, the CTF coefficients matrix is estimated through the application of the adaptive filter algorithm. It is worth noting that the RLS algorithm optimization process discussed in [16] is currently being conducted in the complex domain. The RLS algorithm aims to minimize the sum of the weighted error norm square as

| , | ( 67 ) |
| --- | --- |

where p represents both the adaptation iteration and the frame index, while λ represents the forgetting factor, which imparts weight to the square of the error norm concerning the iteration. Guided by the objective function articulated in ( 67 ), the RLS algorithm is employed for the estimation of the CTF coefficients matrix A. The RLS approach can be succinctly summarized in the subsequent algorithmic routine.

Algorithm 3 CTF estimation using RLS

Input: yd,p, sDAS,p - Initialize RLS forgetting factor λ, weight and inverse of correlation matrix as , where ε is a small positive constant - For each instant of frame, p = 1, 2, …, compute .; ( 68 ).

It should be noted that the estimated matrix of CTF coefficients A changes depending on the processing frame too. It should be noted that the estimated matrix of CTF coefficients A changes depending on the processing frame, and the accuracy improves with an increase in the number of processed frames too.

4.5. Kalman Algorithm Adaptive Filtering Approach

For the third technique, the CTF coefficients matrix is estimated by applying the Kalman filter. It is worth noting that in this thesis, we adapt the Kalman filter as an adaptive filter, instead of using it as a state space control filter. Despite this modification, the primary concept remains unchanged. The process equation of stationary Kalman adaptive filter of each microphone without process noise is described as

| , | ( 69 ) |
| --- | --- |

where wKalman,pm ∈ ℂLх1 signifies the optimal weight vector and has a connection with the CTF coefficients matrix as

| , | ( 70 ) |
| --- | --- |

where Apm denotes the m-th row of A. The measurement equation of stationary Kalman adaptive filter of each microphone is described as

| , | ( 71 ) |
| --- | --- |

where epm denotes the measurement noise for each microphone, and

| , | ( 72 ) |
| --- | --- |

where Rpm is the covariance of measurement noise.

Using the process and measurement equations outlined in ( 69 ) and ( 71 ), the Kalman gain can be derived by minimizing the error covariance matrix [17] . The subsequent algorithmic routine provides a succinct summary of the stationary Kalman adaptive filter approach.

Algorithm 4 CTF estimation using stationary Kalman adaptive filtering

Input: yd,pm, sDAS,p - Initialize estimated Kalman weight, error covariance matrix, Kalman gain and measurement noise covariance matrix as , where η and ρ is a small positive constant - For each microphone, m = 1, 2, …, For each instant of frame, p = 1, 2, …, compute .; ( 73 ).

It is easy to observe that the estimated matrix of CTF coefficients A exhibits variability contingent upon the processing frame, and the accuracy improves with an increase in the number of processed frames too.

4.6. ATF Reconstruction

Once the matrix of CTF coefficients A has been estimated from the three approaches mentioned earlier, we can proceed with producing the ATFs. Our first step is to generate a unit pulse sequence δ(n), which experiences a delay of (L – 1)D points as

| . | ( 74 ) |
| --- | --- |

Subsequently, it is transformed into the STFT domain, resulting in δp. Ultimately, the estimated CTF coefficients are convolved with the transformed unit pulse sequence δp to yield the following signal:

| , | ( 75 ) |
| --- | --- |

where p ∈ [0, PATF]. The estimated RIRs gm(n) n ∈ [0, NATF] can be obtained by applying the inverse STFT to Gpm. Subsequently, the estimated ATFs, represented by vector gm with each element corresponding to different frequency bins, can be obtained by performing a fast Fourier transform (FFT) on the estimated RIRs gm(n). gm is expressed as

| . | ( 76 ) |
| --- | --- |

4.7. Parameters Optimization

While the algorithms outlined above showcase promising outcomes in simulations, it is imperative to recognize the considerable effect that parameters within these algorithms can have on the outcomes. To optimize the performance of the algorithms proposed, we utilize Particle Swarm Optimization (PSO) and its advanced versions [19] to optimize the parameters that are involved.

4.7.1. PSO

The PSO algorithm is a swarm intelligent optimization technique inspired by the flocking of birds and schooling of fish [18] . PSO represents each particle's position as a candidate solution during the exploration of a U-dimensional space. At the t-th update iteration, one particle j among the J particles in the population is characterized by its position and velocity as follows:

| . | ( 77 ) |
| --- | --- |

Let the fitness function  be the one that is required to be minimized. The function accepts a candidate solution in the form of a real vector and produces a real number that represents the fitness value of the given candidate solution. In our case, the candidate solution corresponds to the parameters in our proposed algorithms, and the fitness function can be described as follows:

| , | ( 78 ) |
| --- | --- |

where A(XJ(t)) represents the estimated CTF coefficients matrix obtained from any one of the three methods when the parameters XJ(t) are specified. After calculating the fitness value of the entire population, pbestj(t) and gbest(t) are updated, which are the personal best position of the j-th particle and the global best position in the population, respectively.

|  | ( 79 ) |
| --- | --- |

The velocity and position are then updated using the formulas as below:

| , | ( 80 ) |
| --- | --- |

where win represents the inertia weight, r1 and r2 are random variables that fall within the interval [0, 1] and c1 and c2 denote two positive acceleration coefficients. It is noteworthy that the update process will persist as long as the maximum iteration limit Tmax has not been reached. The PSO algorithm's entire process is presented in Figure 1.

Figure 1 Block diagram of PSO

4.7.2. ASPSO

Although PSO is widely used in the optimization process, it remains limited in its ability to address complicated optimization problems, including premature convergence and insufficient balance between global exploration and local exploitation. To mitigate these challenges, a novel hybrid PSO algorithm using an adaptive strategy (ASPSO) has been developed [19] . It includes four main modifications, namely: inertia weight with chaotic, elite and dimensional learning strategies, adaptive position update strategy and competitive substitution mechanism. These modifications are explained in the following sections.

The inertia weight win plays a key role in harmonizing exploration and exploitation within the search progress. Therefore, the choice of the inertia weight is important. While a linear inertia weight is commonly used, the majority of real-world practical scenarios involve complex non-linear systems. Taking advantage of the randomness, ergodicity and sensitivity inherent in chaotic maps, the C-PSO algorithm incorporates a non-linear approach to adjusting the inertia weight [33] . The formula for calculating inertia weight is

| , | ( 81 ) |
| --- | --- |

where Cin, wmax, wmin and Tmax denote a small positive integer, maximum inertia weight, minimum inertia weight and maximum iteration, respectively.

The basic PSO uses personal and global learning strategies to control the velocity and position updates of the particles. Specifically, all particles use their collective best experiences (pbestj(t) and gbest(t)) to accelerate solution progress. However, this approach can lead to trapping in local optimal when dealing with multimodal features. To mitigate this challenge, [19] introduce elite and dimensional learning strategies. In the elite learning strategy, particles learn from exceptional individuals to increase the diversity of the population. Throughout the search, each particle learns from four personal best positions of different particles pbestj(t) randomly selected from the population. Subsequently, the personal best particle j is compared with the above four particles, and the particle with the best fitness value is retained as the new personal best (Fpbestj(t)). The learning strategy is expressed as

| . | ( 82 ) |
| --- | --- |

An excessive focus on gbest(t) can lead to a rapid diversity in population. To mitigate this potential problem, [19] use the dimensional learning method. By facilitating communication between particles in the dimensional aspect, the mean value provides complementary information, thereby increasing diversity and improving search efficiency. A global particle, denoted Mpbest(t), is defined as

| . | ( 83 ) |
| --- | --- |

Finally, the velocity update equation is changed to:

| . | ( 84 ) |
| --- | --- |

Conventional PSO faces the challenge of achieving an effective balance between global exploration and local exploitation during the search process. The position update law induces particles to consistently converge to their previously determined optimal positions, thereby limiting their ability to explore neighborhoods around the known optimal solution. In response to this constraint, a spiral mechanism has been introduced as a local search operator in the vicinity of the known optimal solution region [34] . Building on this inspiration, an adaptive position update strategy that generates particle positions by dynamically orchestrating a balance between local exploitation and global exploration is proposed in [19] . This strategy is articulated by

| , | ( 85 ) |
| --- | --- |

where Dj represents the distance between the current best position and the j-th particle. The parameter b serves as a constant that determines the shape of the logarithmic spiral, and l is a random number in the range [-1,1]. During each iteration, a ratio βj(t) is calculated by evaluating the fitness value of the current particle in relation to the average fitness value. If βj(t) is small, indicating that the particle is close to the optimal position, there is a need to increase its local exploitation capability. Conversely, if the particle is in a suboptimal position, an update is implemented to increase its global exploration capability, thereby mitigating premature convergence.

Finally, a competitive substitution mechanism is introduced to enhance the performance of PSO [19] . In each iteration, the worst-performing particle is identified and replaced, as defined by

| , | ( 86 ) |
| --- | --- |

where r3∈(0,1) is a random number. During the search process, all particles in the population acquire knowledge from the global best particle gbest(t). Therefore, gbest(t) significant influences the entire population. In a complex search environment, if gbest(t) becomes trapped in a local optimum, the remaining particles tend to converge towards the suboptimal region, leading to premature convergence. Accordingly, a perturbation strategy is built into ASPSO to facilitate the escape of gbest(t) from local optimal. To minimize the time spent on unfavorable directions, a condition is set to trigger the perturbation strategy if gbest(t) fails to update its value after five iterations. The perturbation strategy is described as follows:

| , | ( 87 ) |
| --- | --- |

where r4∈(0,1) is a random number.

The ASPSO algorithm's entire process is presented in Figure 2.

Figure 2 Block diagram of ASPSO

4.8. Summary of Proposed Method

Table 1 summarizes the method proposed in this thesis, describing the resulting output signals obtained at each stage.

Table 1 Flow chart of our proposed method

Flow chart of our proposed method

Step 1: Acquire the microphone signal ym(n) and delay it to get ydm(n) Step 2: Do WPE followed by DAS to ym(n) obtaining sDAS,p Step 3: Estimate the CTF coefficients matrix A via one of the algorithms below - Algorithm 2 CTF estimation using Wiener filtering - Algorithm 3 CTF estimation using RLS - Algorithm 4 CTF estimation using stationary Kalman adaptive filtering Step 4: Do ( 74 ) ~ ( 76 ) to obtain estimated RIRs gm(n) or ATFs gm Step 5: Filter parameters optimization through PSO or ASPSO. Step 6: Applications: MINT for dereverberation, etc.

SIMULATIONS

5.1. Simulation Settings and Parameters

To evaluate the effectiveness of the proposed approach for estimating ATFs, we have employed MINT [21] to perform dereverberation using the estimated RIRs. In addition, we have also compared our approach with the state-of-the-art dereverberation method, WPE [13] , following DAS beamformer. The RIR dataset was generated using the RIR Generator [35] and the dimensions of the room were 5 m × 6 m × 2.5 m. The first sensor of a 30-microphone Uniform Linear Array (ULA) was situated at (1 m, 1.5 m, 1 m) with a 0.02 m gap along the x-axis. The room's layout is presented in Figure 3. At 16 kHz, speech signals were sampled and employed as sources for generating microphone signals, which were convolved with the RIRs of ground truth. The speech sources' location was at (2 m, 2.6 m, 1 m). The simulation verified seven diverse reverberation times, from T60 = 0.4s to 1.6s with an interval of 0.2s. A 1024-sample Hamming window (64 ms) with 75% overlap performed the STFT.

Figure 3 Configuration of the room for simulations

5.2. Simulation Results and Discussions

5.2.1. Without Parameters Optimization

Throughout the sections, the values of the free parameters α, λ, ε, η and ρ were consistently fixed at 0.999, 0.99, 0.01, 0.5 and 0.001, respectively, as they were found to be appropriate for all conditions. The absolute error of the estimated ATF for all frequency bins and the magnitude of the estimated RIR compared to its ground truth values with T60 = 0.6 s for 10-th microphone and all algorithms are displayed in Figure 4. It is worth noting that in the case of blind estimation, it will inevitably encounter the equalization problem that there will be a scale gap between the estimated RIR and the ground-truth RIR. To address this issue, we rescale the estimated RIR with the ratio calculated as the maximum absolute magnitude of the ground-truth RIR divided by the maximum absolute magnitude of the estimated RIR. Figure 4 reveals a minimal absolute error across all frequency bins and a remarkable correspondence between the magnitude of the estimated RIR and its ground-truth counterparts, thus fulfilling our requirements. Table 2 and Figure 5 demonstrate the ME of the estimated ATFs for all algorithms. The algorithms with the lowest ME at every reverberation time are highlighted in red. The ME is formulated as follows:

| , | ( 88 ) |
| --- | --- |

where gm represents the ground-truth ATF of the m-th microphone. The results demonstrate a consistently low ME across all reverberation times, suggesting a favorable outcome.

- Wiener filter

- RLS

- Kalman filter

Figure 4 Absolute error of the estimated ATF and magnitude of the estimated RIR when T60 = 0.6s

Table 2 ME of the estimated ATFs based on the proposed methods in various reverberation times

| T60 (s) method | 0.4 | 0.6 | 0.8 | 1 | 1.2 | 1.4 | 1.6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wiener | 2.8604 | 4.7624 | 6.058 | 7.6621 | 8.8369 | 10.2624 | 12.3786 |
| RLS | 2.9174 | 4.8753 | 6.9862 | 8.4617 | 9.3017 | 12.1247 | 14.6383 |
| Kalman | 2.8649 | 4.7647 | 5.9785 | 7.6486 | 8.9014 | 10.3768 | 12.4370 |

Figure 5 ME of the estimated ATFs based on the proposed methods in various reverberation times

Five types of signals were evaluated for their dereverberation performance metrics using the Perceptual Evaluation of Speech Quality (PESQ) [24] and Signal-to-Distortion Ratio (SDR) [25] . These included MINT dereverberation using RIRs estimated from the proposed method with a Wiener filter, MINT dereverberation using RIRs estimated from the proposed method with an RLS algorithm, MINT dereverberation using RIRs estimated from the proposed method with a Kalman stationary filter, dereverberation using WPE following DAS beamformer and unprocessed signals. Figure 6 depicts the acquired PESQ for seven varying reverberation times. The plot reveals that the three proposed methods outperform WPE following DAS beamformer in yielding a higher PESQ. It is noticeable that as the reverberation time increases, the PESQs of five signals decline. In a similar vein, Figure 7 showcases the SDR obtained for the same seven reverberation times. It is clear from this graph that the proposed three methods continue to achieve higher SDR than WPE following DAS beamformer, which is a convincing outcome. It is also evident that the SDRs of all five signals decrease as the reverberation time increases.

Figure 6 The PESQ values of the processed and unprocessed signals at different reverberation time

Figure 7 The SDR values of the processed and unprocessed signals at different reverberation time

5.2.2. With Parameters Optimization

Table 3 shows the ME of the estimated ATF of the 10-th microphone using the Kalman stationary filter with and without optimization when T60​ is 0.6 seconds. The parameters of the Kalman stationary filter to be optimized are η and ρ. The parameters of the PSO, namely U, J, Tmax, win, c1 and c2, are set to 2, 50, 100, 0.6, 2 and 2, respectively, and the parameters of the ASPSO, namely U, J, Tmax, z1, Cin, wmax, wmin, b, c1 and c2, are set to 2, 50, 100, 0.4, 4, 0.9, 0.4, 0.3, 2 and 2, respectively. Table 3 reveals that when the filter parameters are optimized using either PSO or ASPSO, the ME can reach a lower value, which is a preferable result.

Table 3 ME of the estimated ATFs with and without optimization at T60 = 0.6s

| Kalman filter without parameters optimization | 5.0698 |
| --- | --- |
| Kalman filter with PSO | 4.9672 |
| Kalman filter with ASPSO | 4.9432 |

EXPERIMENTS

6.1. Experiment Settings and Parameters

6.2. Experiment Results and Discussions

CONCLUSIONS AND FUTURE WORK

  - Conclusions

This thesis presents a blind estimation method for the ATF based on the CTF model. Three techniques are developed for estimating CTF coefficient matrices using the Wiener filter, the RLS algorithm and Kalman stationary filter, respectively. By comparing the magnitude of the estimated ATF with the ground truth ATF and the ME over different reverberation times, it is concluded that the proposed method achieves accurate ATF estimation. Furthermore, using PESQ and SDR, we compare the results obtained from the dereverberation signal produced by WPE following DAS beamformer and MINT based on the estimated RIR of our proposed method. The results show a significant advantage of our proposed method over WPE following DAS beamformer. By optimizing the parameters used in the three proposed techniques, one can easily achieve lower ME of the estimated ATFs.

  - Future Work

Although in this thesis we can accurately estimate the ATF for stationary sources, it is important to note that moving sources are more common in practical scenarios. Therefore, our next objective is to address the challenges posed by moving sources by exploiting the process noise characteristics introduced in the Kalman non-stationary filter. Furthermore, all data presented in this article are the results of computer simulations. The inclusion of real experimental data for validation would increase the credibility of our results. Therefore, another future work is the design and implementation of real experiments.

Finally, although several state-of-the-art BSI techniques [5] [6] [7] have been shown to be useful in small reverberation time scenarios, they are still unable to tackle long reverberation time scenarios. Therefore, we will make efforts to modify these techniques using the CTF signal model.

- REFERENCES

- J. Benesty, T. Gänsler, D. R. Morgan, M. M. Sondhi, and S. L. Gay, “Advances in Network and Acoustic Echo Cancellation,” New York: Springer., 2001.

- Y. Huang, J. Benesty, and J. Chen, “A blind channel identification-based two-stage approach to separation and dereverberation of speech signals in a reverberant environment,” IEEE Trans. Speech Audio Process., vol. 13, no. 5, pp. 882–895, 2005.

- S. Araki, R. Mukai, S. Makino, T. Nishikawa, and H. Saruwatari, “The fundamental limitation of frequency domain blind source separation for convolutive mixtures of speech,” IEEE Trans. Audio, Speech, Lang. Process., vol. 14, no. 3, pp. 774–784, May 2006.

- S. Gannot, D. Burshtein, and E. Weinstein, “Signal enhancement using beamforming and nonstationarity with applications to speech,” IEEE Trans. Audio, Speech, Lang. Process., vol. 49, no. 8, pp. 1614–1626, Aug. 2001.

- Y. Huang and J. Benesty, “Adaptive multi-channel least mean square and Newton algorithms for blind channel identification,” Signal Processing, vol. 82, no. 8, pp. 1127–1138, Aug. 2002.

- Y. Huang and J. Benesty, “A class of frequency-domain adaptive approaches to blind multichannel identification,” IEEE Trans. Signal Processing, vol. 51, no. 1, pp. 11–24, Jan. 2003.

- M. K. Hasan, J. Benesty, P. A. Naylor, and D. B. Ward, “Improving robustness of blind adaptive multichannel identification algorithms using constraints,” Proc. European Signal Processing Conference (EUSIPCO), 2005.

- Y. Avargel and I. Cohen, “On multiplicative transfer function approximation in the short-time Fourier transform domain,” IEEE Signal Processing Letters., vol. 14, no. 5, pp. 337–340, 2007.

- Y. Avargel and I. Cohen, “System identification in the short-time Fourier transform domain with crossband filtering,” IEEE Trans. Audio, Speech, Lang. Process., vol. 15, no. 4, pp. 1305–1319, 2007.

- R. Talmon, I. Cohen and S. Gannot, “Relative transfer function identification using convolutive transfer function approximation,” IEEE Trans. Audio, Speech, Lang. Process., vol. 17, no. 4, pp. 546–555, 2009.

- B. Van Den Broeck, A. Bertrand, P. Karsmakers, B. Vanrumste, H. Van hamme and M. Moonen, “Time-domain generalized cross correlation phase transform sound source localization for small microphone arrays,” 2012 5th European DSP Education and Research Conference (EDERC).,pp.76-80, 2012.

- T. Li, Z. Deng, G. Wang and J. Yan, "Time Difference of Arrival Location Method Based on Improved Snake Optimization Algorithm, “2022 IEEE 8th International Conference on Computer and Communications (ICCC) ., pp. 482-486, 2022.

- T. Nakatani, T. Yoshioka, K. Kinoshita, M. Miyoshi and B. -H. Juang, “Speech Dereverberation Based on Variance-Normalized Delayed Linear Prediction,” IEEE Trans. Audio, Speech, Lang. Process., vol. 18, no. 7, pp. 1717–1731, 2010.

- Harry L. Van Trees, “Optimum array processing: Part Iv of detection, estimation, and modulation theory,” New York: Wiley, 2002.

- J. Benesty, S. Makino, J. Chen, Y. Huang, and S. Doclo, “Study of the Wiener filter for noise reduction,” Speech enhancement, pp. 9-41, 2005.

- S. A. U. Islam and D. S. Bernstein, “Recursive Least Squares for Real-Time Implementation,” IEEE Control Systems Magazine, vol. 39, no. 3, pp. 82-85, June 2019.

- R. E. Kalman, “A New Approach to Linear Filtering and Prediction Problems,” Journal of Basic Engineering, 1960.

- J. Kennedy, and R. Eberhart, “Particle swarm optimization,” Proceedings of ICNN’95-International Conference on Neural Networks, IEEE, pp. 1942-1948, 1995.

- Rui ang, Kuangrong Hao, Lei Chen, Tong Wang, and Chunli Jiang, “A novel hybrid particle swarm optimization using adaptive strategy,” Information Sciences, vol. 579, pp. 231-250, 2021.

- Y. Huang and J. Benesty, “Adaptive blind channel identification: Multi-channel least mean square and Newton algorithms,” 2002 IEEE International Conference on Acoustics, Speech, and Signal Processing, pp. II-1637-II-1640, 2002.

- M. Miyoshi, and Y. Kaneda, “Inverse filtering of room acoustics,” IEEE Trans. Audio, Speech, Lang. Process., vol. 36, no. 2, pp. 145-152, Feb. 1988.

- R. Marxer and J. Janer, “A Tikhonov regularization method for spectrum decomposition in low latency audio source separation,” 2012 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP)., pp. 277-280, 2012.

- Q. Trong The and N. Thi Thu Dung, “A Speech Enhancement in Diffuse Noise Field using Minimum Variance Distortionless Response Filter,”  2022 Conference of Russian Young Researchers in Electrical and Electronic Engineering (ElConRus)., pp. 1395-1398, 2022.

- A. W. Rix, J. G. Beerends, M. P. Hollier, and A. P. Hekstra, “Perceptual evaluation of speech quality (PESQ)-a new method for speech quality assessment of telephone networks and codecs,” IEEE International Conference on Acoustics, Speech, and Signal Processing. Proceedings, vol. 2, pp. 749-752, 2001.

- E. Vincent, R. Gribonval, and C. Fevotte, “Performance measurement in blind audio source separation,” IEEE Trans. Audio, Speech, Lang. Process., vol. 14, no. 4, pp. 1462–1469, 2006

- S. Haykin, “Adaptive Filter Theory,” Englewood Cliffs, NJ: Prentice-Hall,, 1996.

- M. R. Portnoff, “Time-frequency representation of digital signals and systems based on short-time Fourier analysis,” IEEE Trans. Signal Process., vol. ASSP-28, no. 1, pp. 55–69, Feb. 1980.

- S. Farkash and S. Raz, “Linear systems in Gabor time-frequency space,” IEEE Trans. Signal Process., vol. 42, no. 3, pp. 611–617, Jan. 1998.

- J. Wexler and S. Raz, “Discrete gabor expansions,” Signal Process., vol. 21, pp. 207–220, Nov. 1990.

- S. Qian and D. Chen, “Discrete Gabor transform,” IEEE Trans. Signal Process., vol. 41, no. 7, pp. 2429–2438, Jul. 1993.

- W. Kellermann, “Analysis and design of multirate systems for cancellation of acoustical echoes, “ International Conference on Acoustics, Speech, and Signal Processing, vol. 5, pp. 2570-2573, 1988.

- D. Gesbert, and P. Duhamel, “Robust blind channel identification and equalization based on multi-step predictors,” International Conference on Acoustics, Speech, and Signal Processing, pp. 3621–3624, 1997.

- H. A. Hefny, and S. S. Azab, “Chaotic particle swarm optimization,” The 7th International Conference on Informatics and Systems (INFOS), pp. 1-8, 2010.

- Seyedali Mirjalili, and Andrew Lewis, “The whale optimization algorithm,” Advances in Engineering Software, vol. 95, pp.51-67, 2016.

- Emanuël Habets, “Room Impulse Response Generator,” Internal Report, pp. 1-17, 2006.

- To do list

4.1. source localization

4.6. ATF reconstruction

Chapter 6 experiments